AUTOSILICON

# RTL-to-GDSII
Automation Framework

Development Plan, System Architecture, Requirements & Validation Strategy

PROJECT VISION Build a reusable, configuration-driven framework that automates a digital design's journey from RTL verification and synthesis through physical implementation, reporting, and supported physical checks—while preserving reproducibility and making PPA trade-offs measurable.

| Document field | Value |
| --- | --- |
| Project name | AutoSilicon |
| Document type | Development document / technical plan |
| Version | 1.0 |
| Status | Proposed baseline; toolchain and PDK compatibility to be verified |
| Initial target | A small sequence detector through an open-source RTL-to-GDSII flow |
| Recommended environment | Ubuntu Linux + Docker |
| Candidate PDK | SkyWater SKY130; GF180MCU may be evaluated as an alternative |

# Document Guide

1. Executive Summary

2. Goals, Scope and Non-Goals

3. System Overview and Data Flow

4. Proposed Technology Stack

5. Functional and Non-Functional Requirements

6. Architecture and Repository Structure

7. Development Phases and Milestones

8. PPA Experimentation and Reporting

9. Verification and Quality Strategy

10. Risks, Constraints and Mitigations

11. Acceptance Criteria

12. Deliverables and GitHub Portfolio Plan

13. First Implementation Checklist

Appendix A. Key Terms

# 1. Executive Summary

AutoSilicon is a software and design-automation project for creating repeatable RTL-to-GDSII implementation runs. It will orchestrate established EDA tools rather than reimplement synthesis or place-and-route algorithms. Its engineering value lies in configuration management, workflow orchestration, validation, error handling, experiment tracking, and clear reports.

The first milestone is deliberately small: use a sequence detector as a known, easy-to-verify RTL design; run functional checks and synthesis; then complete a supported physical-design flow and inspect the resulting layout. After this baseline is stable, the framework can be reused for larger designs such as FlexRISC16.

IMPORTANT DISTINCTION A generated GDSII file is not automatically tape-out-ready. Foundry signoff, PDK-specific rules, design reviews, and any required manufacturing checks must be satisfied separately. The project will clearly label the level of verification actually completed.

# 2. Goals, Scope and Non-Goals

## 2.1 Primary goals

Provide one consistent entry point for launching a configured implementation run.

Automate supported stages: RTL checks, synthesis, floorplanning, power planning, placement, clock-tree synthesis, routing, and report collection.

Keep design-specific settings separate from reusable orchestration code.

Record tool versions, PDK selection, configuration, logs, reports, and output artifacts for every run.

Compare design runs using area, timing, and power metrics where those metrics are available and meaningful.

Make failures visible with stage-level status and actionable log pointers.

Support more than one RTL design without rewriting the framework.

## 2.2 Initial scope

A small sequential RTL design (sequence detector) as the baseline.

Open-source-first toolchain with a documented PDK and flow version.

Python/Tcl/Make orchestration as appropriate to each tool.

CSV or JSON metric export plus a human-readable summary report.

GitHub repository with documentation, example configuration, and lightweight automated tests.

## 2.3 Non-goals for version 1

Creating a new synthesis engine, place-and-route engine, or signoff engine.

Guaranteeing foundry acceptance or manufacturing readiness.

Claiming accurate power results without suitable libraries, constraints, and analysis methodology.

Supporting every PDK or EDA tool from day one.

Replacing detailed manual review of constraints, timing exceptions, physical checks, or tool reports.

# 3. System Overview and Data Flow

The framework coordinates existing tools through a controlled pipeline. Each stage should have explicit inputs, outputs, status, logs, and validation gates.

| Stage | Input | Primary action | Expected output / evidence |
| --- | --- | --- | --- |
| 1. RTL validation | RTL, testbench, parameters | Compile, simulate, run assertions/tests | Pass/fail summary, simulator logs |
| 2. Synthesis | RTL, constraints, libraries | Elaborate and map logic to cells | Gate-level netlist, synthesis reports |
| 3. Floorplan | Netlist, LEF/technology, config | Set core/die, pins and basic physical plan | Floorplan database and reports |
| 4. Power planning | Floorplan, power intent/config | Create supported power structures | Power-grid geometry and checks |
| 5. Placement | Netlist, floorplan, timing constraints | Place standard cells | Placed database, congestion/area reports |
| 6. CTS | Placed design, clock constraints | Build clock distribution network | Clock-tree data and timing reports |
| 7. Routing | CTS design, routing rules | Route signal and clock nets | Routed database, route status |
| 8. Analysis/checks | Routed design, libraries, rules | Timing analysis and supported physical checks | STA, DRC/LVS status where configured |
| 9. Export/report | Final database and reports | Export layout and summarize metrics | GDSII, manifest, CSV/JSON, summary report |

## 3.1 High-level architecture

Command-line interface (CLI): accepts a design, configuration, and requested workflow.

Configuration layer: validates paths, clock targets, I/O constraints, PDK settings, and stage options.

Workflow runner: executes stages in order, captures exit codes and logs, and stops or continues according to policy.

Tool adapters: isolate tool-specific command generation and output parsing.

Artifact manager: stores outputs in unique run directories with a manifest.

Metrics and report engine: extracts supported metrics and produces consistent summaries.

# 4. Proposed Technology Stack

| Area | Candidate tool | Role / notes |
| --- | --- | --- |
| Environment | Ubuntu Linux + Docker | Reproducible development environment; pin image/tool versions where practical. |
| RTL simulation | Icarus Verilog or Verilator | Functional simulation and/or linting; exact choice depends on testbench needs. |
| Synthesis | Yosys | RTL elaboration and logic synthesis. |
| Physical design | OpenROAD | Physical implementation stages supported by the selected flow and platform. |
| PDK / libraries | SkyWater SKY130 | Initial candidate; verify compatible flow scripts, libraries, and rules before committing. |
| Alternative PDK | GF180MCU | Potential alternative after baseline, not a promise of drop-in compatibility. |
| Layout inspection | KLayout | Inspect exported GDSII and geometry. |
| Physical verification | Magic and/or KLayout-based checks | Use only checks supported and correctly configured for the chosen PDK. |
| Automation | Python, Tcl, Make | CLI, orchestration, tool scripts, and developer convenience. |
| Version control / CI | Git, GitHub Actions | Code review, regression tests, and artifact publishing for lightweight jobs. |

TOOLCHAIN GATE Before implementation, verify installation, version compatibility, PDK licensing/terms, platform files, standard-cell libraries, timing corners, and physical verification support. Record exact versions in the repository rather than relying on unpinned latest releases.

# 5. Requirements

## 5.1 Functional requirements

| ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-01 | Select a design and configuration from the CLI. | Must | CLI smoke test |
| FR-02 | Validate required files and configuration before tool execution. | Must | Invalid-config tests |
| FR-03 | Run stages in a defined dependency order. | Must | Integration run and stage log |
| FR-04 | Capture stdout/stderr, exit code, duration, and stage status. | Must | Failure-injection test |
| FR-05 | Keep artifacts isolated by run ID. | Must | Two-run collision test |
| FR-06 | Collect key reports and preserve raw tool outputs. | Must | Artifact manifest check |
| FR-07 | Export available metrics as JSON/CSV. | Should | Schema and parsing tests |
| FR-08 | Generate a human-readable summary report. | Should | Report-generation test |
| FR-09 | Support multiple designs via configuration. | Should | Run two sample designs |
| FR-10 | Run lightweight RTL checks in CI. | Should | GitHub Actions result |

## 5.2 Non-functional requirements

Reproducibility: every run records the configuration, tool versions, selected PDK/platform, and source revision where available.

Maintainability: tool-specific logic is isolated in adapters instead of being scattered through the CLI.

Portability: setup instructions describe supported operating systems and known limitations.

Observability: stage logs and status summaries identify where and why a run failed.

Data integrity: the framework does not silently overwrite previous runs.

Extensibility: new designs, tools, or PDKs can be added with documented interfaces.

Honest reporting: missing metrics are marked unavailable rather than guessed.

# 6. Architecture and Repository Structure

## 6.1 Suggested repository layout

autosilicon/
├── README.md
├── LICENSE
├── pyproject.toml
├── Makefile
├── configs/
│   ├── default.yaml
│   └── designs/
│       └── sequence_detector.yaml
├── designs/
│   └── sequence_detector/
│       ├── rtl/
│       ├── tb/
│       └── README.md
├── src/autosilicon/
│   ├── cli.py
│   ├── config.py
│   ├── workflow.py
│   ├── runner.py
│   ├── artifacts.py
│   ├── metrics.py
│   ├── report.py
│   └── adapters/
│       ├── simulator.py
│       ├── yosys.py
│       └── openroad.py
├── flows/
│   └── platform-specific scripts and templates
├── tests/
│   ├── unit/
│   └── integration/
├── docs/
│   ├── architecture.md
│   ├── setup.md
│   ├── constraints.md
│   └── troubleshooting.md
├── scripts/
└── results/                 # generated locally; normally gitignored

## 6.2 Run directory convention

runs/<design>/<run-id>/
├── manifest.json
├── config/
├── logs/
│   ├── rtl_check.log
│   ├── synthesis.log
│   ├── floorplan.log
│   ├── place.log
│   ├── cts.log
│   └── route.log
├── reports/
├── netlists/
├── databases/
└── layout/
    └── final.gds

Generated outputs should normally be excluded from Git unless a small, intentionally selected sample artifact is useful for demonstrating the project. The manifest should record paths, hashes where useful, timestamps, tool versions, and final status.

## 6.3 CLI concept

autosilicon validate --config configs/designs/sequence_detector.yaml
autosilicon run --config configs/designs/sequence_detector.yaml
 autosilicon report --run-id <run-id>
autosilicon compare --runs <run-a>,<run-b>

These commands are proposed interface examples, not implemented commands yet. Exact syntax may change during development.

# 7. Development Phases and Milestones

| Phase | Estimated duration | Work items | Exit criteria |
| --- | --- | --- | --- |
| 0. Environment validation | 2–5 days | Install or containerize tools; confirm PDK/platform compatibility; run vendor/open-source example flow. | A known-good reference flow runs and versions are documented. |
| 1. Baseline flow | Weeks 1–2 | Sequence detector, testbench, synthesis, physical implementation, layout inspection. | One end-to-end run completes with retained logs and artifacts. |
| 2. Workflow automation | Weeks 3–5 | CLI, config validation, stage runner, logging, status, run directories. | A new run can be started consistently from a single command. |
| 3. Metrics and reports | Weeks 6–8 | Parse reports, export JSON/CSV, generate summary, compare runs. | Metrics are traceable to raw tool reports; missing data is explicit. |
| 4. Robustness and checks | Weeks 9–10 | Failure handling, integration tests, supported DRC/LVS checks, reproducibility improvements. | Expected failures are detected and reported; checks are documented. |
| 5. Multi-design and CI | Weeks 11–12 | Add another small design, lightweight CI, documentation, release assets. | A second design uses the same framework and CI checks pass. |
| 6. Portfolio expansion | After baseline | Adapt flow to FlexRISC16 and explore PPA experiments. | Published results include configuration, limitations, and reproducible evidence. |

Timeline note: these are planning estimates. Tool installation, PDK compatibility, physical verification setup, and timing-constraint debugging can take longer than expected.

# 8. PPA Experimentation and Reporting

The framework should make comparisons useful, fair, and repeatable. Change one or a small number of variables at a time and retain the complete configuration for each run.

## 8.1 Metrics to collect

| Category | Metrics / evidence | Important qualification |
| --- | --- | --- |
| Area | Cell area, core/die area, utilization, buffer count where available | Distinguish cell area from core and die area. |
| Performance | Clock target, setup slack, worst negative slack, critical path delay | Record analysis stage and timing corner; pre-route and post-route results differ. |
| Power | Dynamic/leakage estimates if supported, switching assumptions | Power is only meaningful with suitable libraries, activity assumptions, and analysis setup. |
| Routing | Congestion, signal-metal usage, unrouted nets, route violations | Use tool-native definitions and preserve raw reports. |
| Quality checks | DRC/LVS outcomes and counts where configured | A pass applies only to the selected rules, setup, and tool coverage. |
| Reproducibility | Tool versions, PDK, config hash, source revision, run ID | Capture enough information to reproduce or explain a run. |

## 8.2 Experiment matrix

After a stable baseline, vary a limited number of parameters to investigate trade-offs.

| Experiment | Variable | Question to answer |
| --- | --- | --- |
| A | Clock period target | At what target does timing fail or require substantially more area? |
| B | Core utilization target | How does utilization affect congestion, wire length, and timing? |
| C | RTL implementation variant | Does an equivalent RTL change affect synthesis and physical PPA? |
| D | Pipeline / architecture variant (later) | What is the area/performance trade-off for a larger design? |

Do not claim a universal optimum from a small experiment set. Report the tested configuration, toolchain, PDK, constraints, and limitations alongside every chart.

# 9. Verification and Quality Strategy

## 9.1 Functional verification

Write a specification for the sequence detector, including overlap behavior and reset semantics.

Create directed tests for expected patterns, non-matches, reset, and boundary cases.

Add assertions where supported and test parameter variations if the design is parameterized.

Keep RTL simulation separate from synthesis and physical implementation success.

## 9.2 Automation tests

Unit tests for configuration parsing, path validation, manifest generation, and report parsing.

Failure-injection tests for missing files, invalid settings, and non-zero tool exit codes.

Integration test for the smallest supported end-to-end flow.

Regression comparison that detects unexpected missing metrics or major result changes without assuming every numeric difference is a failure.

CI jobs should initially run lightweight tests; full physical-design runs may be too resource-intensive for standard hosted runners.

## 9.3 Physical checks

Run design-rule and layout-versus-schematic checks only when the selected tool, PDK, and flow are properly configured. Record the exact rule deck and configuration used. A DRC/LVS pass is not a substitute for timing signoff, reliability analysis, or foundry review.

# 10. Risks, Constraints and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Tool and PDK incompatibility | Flow cannot run or produces incomplete checks. | Validate the official reference flow first; pin versions; document supported combinations. |
| Incorrect or incomplete timing constraints | Misleading timing results or false pass. | Review clocks, I/O delays, loads, exceptions, and timing corners; retain constraint files. |
| Misinterpreted power metrics | Unsupported performance claims. | Document analysis assumptions and label unavailable/unreliable estimates. |
| Long runtimes / high resource use | Slow iteration and CI failures. | Start with small designs; separate unit CI from full physical runs. |
| Fragile log parsing | Incorrect metrics or silent data loss. | Prefer structured outputs where available; test parsers; preserve raw reports. |
| False claim of tape-out readiness | Portfolio credibility and technical risk. | Clearly state verification scope and distinguish prototype output from foundry signoff. |
| Generated files overwhelm repository | Large, noisy Git history. | Store run outputs outside source tree or as selected release artifacts; use .gitignore. |

# 11. Acceptance Criteria

A documented environment can run the selected baseline design.

The sequence detector passes its functional testbench before physical implementation.

Synthesis produces a netlist and reports using the chosen platform.

The configured physical flow reaches the documented final stage and exports a layout if supported by the chosen flow.

Every run has a manifest, configuration snapshot, logs, reports, and a clear final status.

Available PPA metrics are exported with units and source-report references.

Invalid configurations and tool failures produce a non-success status and a useful error summary.

At least one second design can be added without modifying core orchestration logic.

Setup, architecture, constraints, limitations, and troubleshooting are documented.

The repository makes no unsupported claims about signoff or manufacturing readiness.

# 12. Deliverables and GitHub Portfolio Plan

## 12.1 Core deliverables

Source repository with clean setup and contribution instructions.

Baseline RTL design and testbench.

Versioned configuration files and toolchain manifest.

Automation scripts and tool adapters.

Sample run logs, reports, and selected layout screenshot or artifact.

PPA comparison report with transparent assumptions.

Architecture diagram and flow diagram.

Known limitations and troubleshooting guide.

## 12.2 Suggested release milestones

| Release | Name | Definition of done |
| --- | --- | --- |
| v0.1 | Baseline Flow | A documented reference flow completes on the selected environment. |
| v0.2 | Automation CLI | Configuration validation, stage runner, logs, and run isolation work. |
| v0.3 | Metrics & Reports | Metric export and readable run summaries are available. |
| v0.4 | Quality & CI | Unit tests, failure handling, and lightweight CI are in place. |
| v1.0 | Multi-Design Framework | Two designs use the framework; documentation and example results are published. |

## 12.3 Portfolio presentation

The README should lead with the problem, supported toolchain, architecture, quick-start commands, and a sample result. Include a diagram, a short demonstration, PPA charts with methodology, and explicit limitations. The strongest portfolio story is not simply 'generated GDSII'; it is 'built a reproducible flow, validated its stages, and measured implementation trade-offs.'

# 13. First Implementation Checklist

☐ Choose the initial environment: recommended Ubuntu + Docker.

☐ Select one PDK/platform and confirm its supported reference flow.

☐ Record exact versions of the simulator, Yosys, OpenROAD, layout tools, and PDK.

☐ Create the repository skeleton and initial README.

☐ Implement and test the sequence detector specification.

☐ Run the official/minimal platform flow before adding custom orchestration.

☐ Save a known-good baseline run and its reports.

☐ Define the configuration schema and manifest fields.

☐ Implement stage execution and log capture.

☐ Add metric extraction only after raw reports are understood.

☐ Create a reproducible demo and document remaining limitations.

# Appendix A. Key Terms

| Term | Meaning |
| --- | --- |
| RTL | Register-transfer level: a hardware description of registers and the logic between them. |
| Synthesis | Transformation of RTL into a technology-mapped logic netlist. |
| PDK | Process design kit: technology files, libraries, and rules used by EDA tools. |
| STA | Static timing analysis: timing verification using paths, constraints, and cell/wire delays. |
| CTS | Clock-tree synthesis: construction of the clock distribution network. |
| PPA | Power, performance, and area—the main implementation trade-off dimensions. |
| GDSII | A common stream format for representing integrated-circuit layout geometry. |
| DRC | Design-rule checking against the selected physical rule deck. |
| LVS | Layout-versus-schematic/netlist comparison, according to the supported verification flow. |
| Tape-out | Release of a design package for fabrication after applicable signoff and manufacturing requirements. |

NEXT ACTION Start by validating a known-good SKY130 reference flow inside Ubuntu + Docker. Once that succeeds, wrap it with AutoSilicon's configuration, run management, logging, and reporting layers. This reduces the risk of debugging the EDA toolchain and the new automation framework at the same time.